TASM TABLES  Version 2.8                                      Page 1

6502 INSTRUCTIONS AND ADDRESSING MODES

The acceptable 6502 opcode mnemonics for TASM are as follows:

ADC  AND  ASL  BCC  BCS  BEQ  BNE  BMI  BPL  BVC  BVS  BIT

BRK  CLC  CLD  CLI  CLV  CMP  CPX  CPY  DEC  DEX  DEY  EOR

INC  INX  INY  JMP  JSR  LDA  LDX  LDY  LSR  NOP  ORA  PHA

PHP  PLA  PLP  ROL  ROR  RTI  RTS  SBC  SEC  SED  SEI  STA

STX  STY  TAX  TAY  TSX  TXA  TXS  TYA

TASM  also  supports the following instructions that are part of the

Rockwell  R65C02  and  R65C00/21  microprocessor  instruction  sets.

Those  that  are  marked  as  set A are applicable to the R65C02 and

those marked as set B are  applicable  to  the  R65C00/21  (A+B  for

both):

Mnemonic        Description                Address Mode  Set

---------------------------------------------------------------

ADC             Add with carry             (IND)         A

AND             And memory with A          (IND)         A

BIT             Test memory bits with A    ABS,X         A

BIT             Test memory bits with A    ZP,X          A

BIT             Test memory bits with A    IMM           A

CMP             Compare memory with A      (IND)         A

DEC             Decrement A                A             A

EOR             Exclusive OR memory with A (IND)         A

INC             Increment A                A             A

JMP             Jump                       (ABS,X)       A

LDA             Load A with memory         (IND)         A

ORA             OR A with memory           (IND)         A

SBC             Subtract memory form A     (IND)         A

STA             Store A in memory          (IND)         A

STZ             Store zero                 ABS           A

STZ             Store zero                 ABS,X         A

STZ             Store zero                 ZP            A

STZ             Store zero                 ZP,X          A

TRB             Test and reset memory bit  ABS           A

TRB             Test and reset memory bit  ZP            A

TSB             Test and set memory bit    ABS           A

TSB             Test and set memory bit    ZP            A

BRA             Branch Always              REL           A+B

BBR0            Branch on Bit 0 Reset      ZP,REL        A+B

BBR1            Branch on Bit 1 Reset      ZP,REL        A+B

BBR2            Branch on Bit 2 Reset      ZP,REL        A+B

BBR3            Branch on Bit 3 Reset      ZP,REL        A+B

BBR4            Branch on Bit 4 Reset      ZP,REL        A+B

BBR5            Branch on Bit 5 Reset      ZP,REL        A+B

BBR6            Branch on Bit 6 Reset      ZP,REL        A+B

BBR7            Branch on Bit 7 Reset      ZP,REL        A+B

BBS0            Branch on Bit 0 Set        ZP,REL        A+B

BBS1            Branch on Bit 1 Set        ZP,REL        A+B

BBS2            Branch on Bit 2 Set        ZP,REL        A+B

TASM TABLES  Version 2.8                                      Page 2

BBS3            Branch on Bit 3 Set        ZP,REL        A+B

BBS4            Branch on Bit 4 Set        ZP,REL        A+B

BBS5            Branch on Bit 5 Set        ZP,REL        A+B

BBS6            Branch on Bit 6 Set        ZP,REL        A+B

BBS7            Branch on Bit 7 Set        ZP,REL        A+B

MUL             Multiply                   Implied       B

PHX             Push Index X               Implied       A+B

PHY             Push Index Y               Implied       A+B

PLX             Pull Index X               Implied       A+B

PLY             Pull Index Y               Implied       A+B

RMB0            Reset Memory Bit 0         ZP            A+B

RMB1            Reset Memory Bit 1         ZP            A+B

RMB2            Reset Memory Bit 2         ZP            A+B

RMB3            Reset Memory Bit 3         ZP            A+B

RMB4            Reset Memory Bit 4         ZP            A+B

RMB5            Reset Memory Bit 5         ZP            A+B

RMB6            Reset Memory Bit 6         ZP            A+B

RMB7            Reset Memory Bit 7         ZP            A+B

SMB0            Set   Memory Bit 0         ZP            A+B

SMB1            Set   Memory Bit 1         ZP            A+B

SMB2            Set   Memory Bit 2         ZP            A+B

SMB3            Set   Memory Bit 3         ZP            A+B

SMB4            Set   Memory Bit 4         ZP            A+B

SMB5            Set   Memory Bit 5         ZP            A+B

SMB6            Set   Memory Bit 6         ZP            A+B

SMB7            Set   Memory Bit 7         ZP            A+B

Addressing  modes  are denoted as follows:

ABS               Absolute

ZP                Zero Page

ABS,X             Absolute X

ZP,X              Zero Page X

ABS,Y             Absolute Y

ZP,Y              Zero Page Y

A                 Accumulator

(IND,X)           Indirect X

(IND),Y           Indirect Y

(IND)             Indirect

#IMM              Immediate

REL               Relative (Branch instructions only)

ZP,REL            Zero Page, Relative

Implied           Implied

Note  that Zero Page addressing can not be explicitly requested.  It

is used if the value of the operand is  representable  in  a  single

byte for the applicable statements.

The '-x' command line option can be  used  to  enable  the  extended

instructions.  A  '-x'  with  no  digit  following  will  enable the

standard set plus both extended sets.  The 6502 version of TASM uses

three bits in the instruction class  mask  to  determine  whether  a

TASM TABLES  Version 2.8                                      Page 3

given  instruction  is enabled or not.  Bit 0 enables the basic set,

bit 1 enables set A (R65C02) and bit 2 enables  set  B  (R65C00/21).

The following table shows various options:

Class Mask        Enabled Instructions

BASIC   R65C02  R65C00/21

--------------------------------------------

1               yes     no      no

2               no      yes     no

3               yes     yes     no

4               no      no      yes

5               yes     no      yes

6               no      yes     yes

7               yes     yes     yes

Thus,  to enable the basic set plus the R65C02 instructions,  invoke

the '-x3' command line option.

See manufacturer's data for  a  more  complete  description  of  the

meaning    of     the     mnemonics    and     addressing     modes.

TASM TABLES  Version 2.8                                      Page 4

8048 INSTRUCTIONS AND ADDRESSING MODES

The following list shows the acceptable opcode mnemonics  and  their

corresponding  operand  formats for the 8048 version of TASM.  Where

'Rn' is seen,  R0 through R7  may  be  substituted.  Other  symbolic

fields are as follows:

SYMBOLIC            DESCRIPTION

-----------------------------------------------

<addr8>             Absolute address (8 bits)

<addr11>            Absolute address (11 bits)

<immed>             Immediate data

Any valid TASM expression can appear in the  place  of  any  of  the

above symbolics.

The lines that are marked with an (8041),  (8022),  or (8021) on the

far  right are extended instructions that are available only if a -x

option has  been  invoked  on  the  command  line.  The  classes  of

instructions  (and their bit assignment in the class mask) are shown

below:

BIT     PROCESSOR

-------------------------------

0       8X48, 8035, 8039, 8049

1       8X41A

2       8022

3       8021

Thus, to enable the basic 8048 set plus the 8022 set, a -x5 could be

used on the command line.

Note that some of the base instructions should be disabled  for  the

8041, 8022, and 8021, but are not.

OPCODE  OPERANDS        DESCRIPTION

-------------------------------------------------------------------

ADD     A,Rn            Add Register to Acc

ADD     A,@R0           Add Indirect RAM to Acc

ADD     A,@R1           Add Indirect RAM to Acc

ADD     A,#<immed>      Add Immediate data to Acc

ADDC    A,Rn            Add Register to Acc with carry

ADDC    A,@R0           Add Indirect RAM to Acc with carry

ADDC    A,@R1           Add Indirect RAM to Acc with carry

ADDC    A,#<immed>      Add Immediate data to Acc with carry

ANL     A,Rn            AND Register to Acc

ANL     A,@R0           AND Indirect RAM to Acc

ANL     A,@R1           AND Indirect RAM to Acc

ANL     A,#<immed>      AND Immediate data to Acc

ANL     BUS,#<immed>    AND Immediate data to BUS

ANL     P1,#<immed>     AND Immediate data to port P1

ANL     P2,#<immed>     AND Immediate data to port P2

ANLD    P4,A            AND Acc to Expander port P4

TASM TABLES  Version 2.8                                      Page 5

ANLD    P5,A            AND Acc to Expander port P5

ANLD    P6,A            AND Acc to Expander port P6

ANLD    P7,A            AND Acc to Expander port P7

CALL    <addr11>        Call subroutine

CLR     A               Clear Acc

CLR     C               Clear Carry

CLR     F0              Clear Flag 0

CLR     F1              Clear Flag 1

CPL     A               Complement Acc

CPL     C               Complement Carry

CPL     F0              Complement Flag F0

CPL     F1              Complement Flag F1

DA      A               Decimal adjust Acc

DEC     A               Decrement Acc

DEC     Rn              Decrement Register

DIS     I               Disable Interrupts

DIS     TCNTI           Disable Timer/Counter Interrupt

DJNZ    Rn,<addr8>      Decrement Register and Jump if nonzero

EN      DMA             Enable DMA                           (8041)

EN      FLAGS           Enable Flags                         (8041)

EN      I               Enable External Interrupt

EN      TCNTI           Enable Timer/Counter Interrupt

ENT0    CLK             Enable Clock Output

IN      A,DBB           Input Data Bus to Acc                (8041)

IN      A,P0            Input Port 0 to Acc                  (8021)

IN      A,P1            Input Port 1 to Acc

IN      A,P2            Input Port 2 to Acc

INC     A               Increment Acc

INC     Rn              Increment Register

INC     @R0             Increment Indirect RAM

INC     @R1             Increment Indirect RAM

INS     A,BUS           Strobed Input of Bus to Acc

JB0     <addr8>         Jump if Acc bit 0 is set

JB1     <addr8>         Jump if Acc bit 1 is set

JB2     <addr8>         Jump if Acc bit 2 is set

JB3     <addr8>         Jump if Acc bit 3 is set

JB4     <addr8>         Jump if Acc bit 4 is set

JB5     <addr8>         Jump if Acc bit 5 is set

JB6     <addr8>         Jump if Acc bit 6 is set

JB7     <addr8>         Jump if Acc bit 7 is set

JMP     <addr11>        Jump

JC      <addr8>         Jump if Carry is set

JF0     <addr8>         Jump if Flag F0 is set

JF1     <addr8>         Jump if Flag F1 is set

TASM TABLES  Version 2.8                                      Page 6

JNC     <addr8>         Jump if Carry is clear

JNI     <addr8>         Jump if Interrupt input is clear

JNIBF   <addr8>         Jump if IBF is clear                 (8041)

JNT0    <addr8>         Jump if T0 is clear

JNT1    <addr8>         Jump if T1 is clear

JNZ     <addr8>         Jump if Acc is not zero

JOBF    <addr8>         Jump if OBF is set                   (8041)

JTF     <addr8>         Jump if Timer Flag is set

JT0     <addr8>         Jump if T0 pin is high

JT1     <addr8>         Jump if T1 pin is high

JZ      <addr8>         Jump if Acc is zero

JMPP    @A              Jump Indirect (current page)

MOV     A,PSW           Move PSW to Acc

MOV     A,Rn            Move Register to Acc

MOV     A,T             Move Timer/Counter to Acc

MOV     A,@R0           Move Indirect RAM to Acc

MOV     A,@R1           Move Indirect RAM to Acc

MOV     A,#<immed>      Move Immediate data to Acc

MOV     PSW,A           Move Acc to PSW

MOV     Rn,A            Move Acc to Register

MOV     Rn,#<immed>     Move Immediate data to Register

MOV     STS,A           Move Acc to STS                      (8041)

MOV     T,A             Move Acc to Timer/Counter

MOV     @R0,A           Move Acc to Indirect RAM

MOV     @R1,A           Move Acc to Indirect RAM

MOV     @R0,#<immed>    Move Immediate data to Indirect RAM

MOV     @R1,#<immed>    Move Immediate data to Indirect RAM

MOVD    A,P4            Move half-byte Port 4 to Acc (lower nibble)

MOVD    A,P5            Move half-byte Port 5 to Acc (lower nibble)

MOVD    A,P6            Move half-byte Port 6 to Acc (lower nibble)

MOVD    A,P7            Move half-byte Port 7 to Acc (lower nibble)

MOVD    P4,A            Move lower nibble of Acc to Port 4

MOVD    P5,A            Move lower nibble of Acc to Port 5

MOVD    P6,A            Move lower nibble of Acc to Port 6

MOVD    P7,A            Move lower nibble of Acc to Port 7

MOVP    A,@A            Move Indirect Program data to Acc

MOVP3   A,@A            Move Indirect Program data to Acc (page 3)

MOVX    A,@R0           Move Indirect External RAM to Acc

MOVX    A,@R1           Move Indirect External RAM to Acc

MOVX    @R0,A           Move Acc to Indirect External RAM

MOVX    @R1,A           Move Acc to Indirect External RAM

NOP                     No operation

ORL     A,Rn            OR Register to Acc

ORL     A,@R0           OR Indirect RAM to Acc

ORL     A,@R1           OR Indirect RAM to Acc

ORL     A,#<immed>      OR Immediate data to Acc

ORL     BUS,#<immed>    OR Immediate data to BUS

ORL     P1,#<immed>     OR Immediate data to port P1

ORL     P2,#<immed>     OR Immediate data to port P2

TASM TABLES  Version 2.8                                      Page 7

ORLD    P4,A            OR lower nibble of Acc with P4

ORLD    P5,A            OR lower nibble of Acc with P5

ORLD    P6,A            OR lower nibble of Acc with P6

ORLD    P7,A            OR lower nibble of Acc with P7

OUTL    BUS,A           Output Acc to Bus

OUT     DBB,A           Output Acc to DBB                    (8041)

OUTL    P0,A            Output Acc to Port P0                (8021)

OUTL    P1,A            Output Acc to Port P1

OUTL    P2,A            Output Acc to Port P2

RAD                     Move A/D Converter to Acc            (8022)

RET                     Return from subroutine

RETI                    Return from Interrupt w/o PSW restore(8022)

RETR                    Return from Interrupt w/  PSW restore

RL      A               Rotate Acc Left

RLC     A               Rotate Acc Left through Carry

RR      A               Rotate Acc Right

RRC     A               Rotate Acc Right through Carry

SEL     AN0             Select Analog Input 0                (8022)

SEL     AN1             Select Analog Input 1                (8022)

SEL     MB0             Select Memory Bank 0

SEL     MB1             Select Memory Bank 1

SEL     RB0             Select Register Bank 0

SEL     RB1             Select Register Bank 1

STOP    TCNT            Stop Timer/Counter

STRT    CNT             Start Counter

STRT    T               Start Timer

SWAP    A               Swap nibbles of Acc

XCH     A,Rn            Exchange Register with Acc

XCH     A,@R0           Exchange Indirect RAM with Acc

XCH     A,@R1           Exchange Indirect RAM with Acc

XCHD    A,@R0           Exchange lower nibble of Indirect RAM w/ Acc

XCHD    A,@R1           Exchange lower nibble of Indirect RAM w/ Acc

XRL     A,Rn            Exclusive OR Register to Acc

XRL     A,@R0           Exclusive OR Indirect RAM to Acc

XRL     A,@R1           Exclusive OR Indirect RAM to Acc

XRL     A,#<immed>      Exclusive OR Immediate data to Acc

See manufacturer's data for  a  more  complete  description  of  the

meaning    of     the     mnemonics    and     addressing     modes.

TASM TABLES  Version 2.8                                      Page 8

8051 INSTRUCTIONS AND ADDRESSING MODES

The following list shows the acceptable opcode mnemonics  and  their

corresponding  operand  formats for the 8051 version of TASM.  Where

'Rn' is seen,  R0 through R7  may  be  substituted.  Other  symbolic

fields are as follows:

SYMBOLIC            DESCRIPTION

-----------------------------------------------

<addr11>            Absolute address (11 bits)

<addr16>            Absolute address (16 bits)

<bit>               Bit address

<immed>             Immediate data

<direct>            Direct RAM address

<rel>               Relative address

Any valid TASM expression can appear in the  place  of  any  of  the

above symbolics.

OPCODE  OPERAND             DESCRIPTION

--------------------------------------------------------------------

ACALL   <addr11>            Absolute Call

ADD     A,Rn                Add Register to Acc

ADD     A,@R0               Add Indirect RAM to Acc

ADD     A,@R1               Add Indirect RAM to Acc

ADD     A,#<immed>          Add Immediate data to Acc

ADD     A,<direct>          Add Direct RAM to Acc

ADDC    A,Rn                Add Register to Acc with carry

ADDC    A,@R0               Add Indirect RAM to Acc with carry

ADDC    A,@R1               Add Indirect RAM to Acc with carry

ADDC    A,#<immed>          Add Immediate data to Acc with carry

ADDC    A,<direct>          Add Direct RAM to Acc with carry

AJMP    <addr11>            Absolute Jump

ANL     A,Rn                AND Register and Acc

ANL     A,@R0               AND Indirect RAM and Acc

ANL     A,@R1               AND Indirect RAM and Acc

ANL     A,#<immed>          AND Immediate data and Acc

ANL     A,<direct>          AND Direct RAM and Acc

ANL     C,/<direct>         AND Complement of direct bit to Carry

ANL     C,<direct>          AND direct bit to Carry

ANL     <direct>,A          AND Acc to direct RAM

ANL     <direct>,#<immed>   AND Immediate data and direct RAM

CJNE    A,#<immed>,<rel>    Compare Immediate to Acc   and JNE

CJNE    A,<direct>,<rel>    Compare direct RAM to Acc and JNE

CJNE    Rn,#<immed>,<rel>   Compare Immediate to Register and JNE

CJNE    @R0,#<immed>,<rel>  Compare Immediate to Indirect RAM and JNE

CJNE    @R1,#<immed>,<rel>  Compare Immediate to Indirect RAM and JNE

CLR     A                   Clear Accumulator

CLR     C                   Clear Carry

CLR     <direct>            Clear Direct RAM

CPL     A                   Complement Accumulator

TASM TABLES  Version 2.8                                      Page 9

CPL     C                   Complement Carry

CPL     <direct>            Complement Direct RAM

DA      A                   Decimal Adjust Accumulator

DEC     A                   Decrement Acc

DEC     Rn                  Decrement Register

DEC     @R0                 Decrement Indirect RAM

DEC     @R1                 Decrement Indirect RAM

DEC     <direct>            Decrement Direct RAM

DIV     AB                  Divide Acc by B

DJNZ    Rn,<rel>            Decrement Register and JNZ

DJNZ    <direct>,<rel>      Decrement Direct RAM and JNZ

INC     A                   Increment Acc

INC     Rn                  Increment Register

INC     @R0                 Increment Indirect RAM

INC     @R1                 Increment Indirect RAM

INC     DPTR                Increment Data Pointer

INC     <direct>            Increment Direct RAM

JB      <bit>,<rel>         Jump if Bit is set

JBC     <bit>,<rel>         Jump if Bit is set & clear Bit

JC      <rel>               Jump if Carry is set

JMP     @A+DPTR             Jump indirect relative to Data Pointer

JNB     <bit>,<rel>         Jump if Bit is clear

JNC     <rel>               Jump if Carry is clear

JNZ     <rel>               Jump if Acc is not zero

JZ      <rel>               Jump if Acc is zero

LCALL   <addr16>            Long Subroutine Call

LJMP    <addr16>            Long Jump

MOV     A,Rn                Move Register to Acc

MOV     A,@R0               Move Indirect RAM to Acc

MOV     A,@R1               Move Indirect RAM to Acc

MOV     A,#<immed>          Move Immediate data to Acc

MOV     A,<direct>          Move direct RAM to Acc

MOV     C,<bit>             Move bit to Acc

MOV     DPTR,#<immed>       Move immediate data to Data Pointer

MOV     Rn,A                Move Acc to Register

MOV     Rn,#<immed>         Move Immediate data to Register

MOV     Rn,<direct>         Move Direct RAM to Register

MOV     @R0,A               Move Acc to Indirect RAM

MOV     @R1,A               Move Acc to Indirect RAM

MOV     @R0,#<immed>        Move Immediate data to Indirect RAM

MOV     @R1,#<immed>        Move Immediate data to Indirect RAM

MOV     @R0,<direct>        Move Direct RAM to Indirect RAM

MOV     @R1,<direct>        Move Direct RAM to Indirect RAM

MOV     <direct>,A          Move Acc to Direct RAM

MOV     <bit>,C             Move Carry to Bit

MOV     <direct>,Rn         Move Register to Direct RAM

MOV     <direct>,@R0        Move Indirect RAM to Direct RAM

MOV     <direct>,@R1        Move Indirect RAM to Direct RAM

MOV     <direct>,#<immed>   Move Immediate data to Direct RAM

TASM TABLES  Version 2.8                                     Page 10

MOV     <direct>,<direct>   Move Direct RAM to Direct RAM

MOVC    A,@A+DPTR           Move code byte relative to DPTR to Acc

MOVC    A,@A+PC             Move code byte relative to PC to Acc

MOVX    A,@R0               Move external RAM to Acc

MOVX    A,@R1               Move external RAM to Acc

MOVX    A,@DPTR             Move external RAM to Acc (16 bit addr)

MOVX    @R0,A               Move Acc to external RAM

MOVX    @R1,A               Move Acc to external RAM

MOVX    @DPTR,A             Move Acc to external RAM (16 bit addr)

MUL     AB                  Multiply Acc by B

NOP                         No operation

ORL     A,Rn                OR Register and Acc

ORL     A,@R0               OR Indirect RAM and Acc

ORL     A,@R1               OR Indirect RAM and Acc

ORL     A,#<immed>          OR Immediate data and Acc

ORL     A,<direct>          OR Direct RAM and Acc

ORL     C,/<direct>         OR Complement of direct bit to Carry

ORL     C,<direct>          OR direct bit to Carry

ORL     <direct>,A          OR Acc to direct RAM

ORL     <direct>,#<immed>   OR Immediate data and direct RAM

POP     <direct>            Pop  from Stack and put in Direct RAM

PUSH    <direct>            Push from Direct RAM to Stack

RET                         Return from subroutine

RETI                        Return from Interrupt

RL      A                   Rotate Acc left

RLC     A                   Rotate Acc left through Carry

RR      A                   Rotate Acc right

RRC     A                   Rotate Acc right through Carry

SETB    C                   Set the Carry Bit

SETB    <bit>               Set Direct Bit

SJMP    <rel>               Short jump

SUBB    A,Rn                Subtract Register from Acc with Borrow

SUBB    A,@R0               Subtract Indirect RAM from Acc w/ Borrow

SUBB    A,@R1               Subtract Indirect RAM from Acc w/ Borrow

SUBB    A,#<immed>          Subtract Immediate data from Acc w/ Borrow

SUBB    A,<direct>          Subtract Direct RAM from Acc w/ Borrow

SWAP    A                   Swap nibbles of Acc

XCH     A,Rn                Exchange Acc with Register

XCH     A,@R0               Exchange Acc with Indirect RAM

XCH     A,@R1               Exchange Acc with Indirect RAM

XCH     A,<direct>          Exchange Acc with Direct RAM

XCHD    A,@R0               Exchange Digit in Acc with Indirect RAM

XCHD    A,@R1               Exchange Digit in Acc with Indirect RAM

TASM TABLES  Version 2.8                                     Page 11

XRL     A,Rn                Exclusive OR Register and Acc

XRL     A,@R0               Exclusive OR Indirect RAM and Acc

XRL     A,@R1               Exclusive OR Indirect RAM and Acc

XRL     A,#<immed>          Exclusive OR Immediate data and Acc

XRL     A,<direct>          Exclusive OR Direct RAM and Acc

XRL     <direct>,A          Exclusive OR Acc to direct RAM

XRL     <direct>,#<immed>   Exclusive OR Immediate data and direct RAM

Note  that  the above tables do not automatically define the various

mnemonics that may be  used  for  addressing  the  special  function

registers  of  the  8051.  The  user  may  wish  to set up a file of

equates (EQU's) that can be included in the  source  file  for  this

purpose.  The following illustrates some of the appropriate equates:

P0      .equ    080H    ;Port 0

SP      .equ    081H    ;Stack pointer

DPL     .equ    082H

DPH     .equ    083H

PCON    .equ    087H

TCON    .equ    088H

TMOD    .equ    089H

TL0     .equ    08AH

TL1     .equ    08BH

TH0     .equ    08CH

TH1     .equ    08DH

P1      .equ    090H    ;Port 1

SCON    .equ    098H

SBUF    .equ    099H

P2      .equ    0A0H    ;Port 2

IEC     .equ    0A8H

P3      .equ    0B0H    ;Port 3

IPC     .equ    0B8H

PSW     .equ    0D0H

ACC     .equ    0E0H    ;Accumulator

B       .equ    0F0H    ;Secondary Accumulator

;Now some bit addresses

P0.0    .equ    080H    ;Port 0 bit 0

P0.1    .equ    081H    ;Port 0 bit 1

P0.2    .equ    082H    ;Port 0 bit 2

P0.3    .equ    083H    ;Port 0 bit 3

P0.4    .equ    080H    ;Port 0 bit 4

P0.5    .equ    081H    ;Port 0 bit 5

P0.6    .equ    082H    ;Port 0 bit 6

P0.7    .equ    083H    ;Port 0 bit 7

ACC.0   .equ    0E0H    ;Acc bit 0

ACC.1   .equ    0E1H    ;Acc bit 1

ACC.2   .equ    0E2H    ;Acc bit 2

ACC.3   .equ    0E3H    ;Acc bit 3

ACC.4   .equ    0E4H    ;Acc bit 4

ACC.5   .equ    0E5H    ;Acc bit 5

ACC.6   .equ    0E6H    ;Acc bit 6

ACC.7   .equ    0E7H    ;Acc bit 7

See  the   manufacturer's   data  sheets   for   more   information.

TASM TABLES  Version 2.8                                     Page 12

8085 INSTRUCTIONS AND ADDRESSING MODES

The following list shows the acceptable opcode mnemonics  and  their

corresponding  operand  formats for the 8085  version of TASM.   The

following symbols are used in the table:

SYMBOLIC            DESCRIPTION

-----------------------------------------------

<addr>              Absolute address (16 bits)

<data>              Immediate data (8 bits)

<data16>            Immediate data (16 bits)

<reg>               Register (A,B,C,D,E,H,L)

<rp>                Register pair (B,D,H,SP)

<port>              Port address (0-255)

<int>               Interrupt level (0 - 7)

Any valid TASM expression can appear in the  place  of  any  of  the

above symbolics except <reg>, <rp> and <int>.

OPCODE  OPERAND        DESCRIPTION

--------------------------------------------------------------------

ACI      <data>         Add immediate to A with carry

ADC      <reg>          Add <reg> to A with carry

ADC      M              Add indirect memory (HL) with carry

ADD      <reg>          Add <reg> to A

ADD      M              Add indirect memory (HL) to A

ADI      <data>         Add immediate to A

ANA      <reg>          And register with A

ANA      M              And indirect memory (HL) to A

ANI      <data>         And immediate to A

CALL     <addr>         Call subroutine at <addr>

CC       <addr>         Call subroutine if carry set

CNC      <addr>         Call subroutine if carry clear

CZ       <addr>         Call subroutine if zero

CNZ      <addr>         Call subroutine if non zero

CP       <addr>         Call subroutine if positive

CM       <addr>         Call subroutine if negative

CPE      <addr>         Call subroutine if even parity

CPO      <addr>         Call subroutine if odd  parity

CMA                     Complement A

CMC                     Complemennt carry

CMP      <reg>          Compare register with A

CMP      M              Compare indirect memory (HL) with A

CPI      <data>         Compare immediate data with A

DAA                     Decimal adjust A

DAD      <rp>           Add register pair to HL

DCR      <reg>          Decrement register

DCR      M              Decrement indirect memory (HL)

DCX      <rp>           Decrement register pair

DI                      Disable interrupts

EI                      Enable interrupts

HLT                     Halt

TASM TABLES  Version 2.8                                     Page 13

IN       <port>         Input on port

INR      <reg>          Increment register

INR      M              Increment indirect memory (HL)

INX      <rp>           Increment register pair

JMP      <addr>         Jump

JC       <addr>         Jump if carry set

JNC      <addr>         Jump if carry clear

JZ       <addr>         Jump if zero

JNZ      <addr>         Jump if not zero

JM       <addr>         Jump if minus

JP       <addr>         Jump if plus

JPE      <addr>         Jump if parity even

JPO      <addr>         Jump if parity odd

LDA      <addr>         Load A direct from memory

LDAX     B              Load A indirect from memory using BC

LDAX     D              Load A indirect from memory using DE

LHLD     <addr>         Load HL direct from memory

LXI      <rp>,<data16>  Load register pair with immediate data

MOV      <reg>,<reg>    Move register to register

MOV      <reg>,M        Move indirect memory (HL) to register

MVI      <reg>,<data>   Move immediate data to register

NOP                     No operation

ORA      <reg>          Or register with A

ORA      M              Or indirect memory (HL) with A

ORI      <data>         Or immediate data to A

OUT      <port>         Ouput to port

PCHL                    Jump to instruction at (HL)

POP      <rp>           Pop  register pair (excluding SP) from stack

PUSH     <rp>           Push register pair (excluding SP) onto stack

POP      PSW            Pop  PSW from stack

PUSH     PSW            Pop  PSW onto stack

RAL                     Rotate A left  with carry

RAR                     Rotate A right with carry

RLC                     Rotate A left  with branch carry

RRC                     Rotate A right with branch carry

RET                     Return from subroutine

RZ                      Return if zero

RNZ                     Return if non zero

RC                      Return if carry set

RNC                     Return if carry clear

RM                      Return if minus

RP                      Return if plus

RPE                     Return if parity even

RPO                     Return if parity odd

RIM                     Read interrupt mask

RST      <int>          Restart at vector <int>

TASM TABLES  Version 2.8                                     Page 14

SBB      <reg>          Subtract <reg> from A         with borrow

SBB      M              Subtract indirect memory (HL) with borrow

SBI      <data>         Subtract immediate from A     with borrow

SUB      <reg>          Subtract <reg> from A

SUB      M              Subtract indirect memory (HL) from A

SUI      <data>         Subtract immediate from A

SHLD     <addr>         Store HL

SIM                     Store Interrupt mask

SPHL                    Exchange SP with HL

STA      <addr>         Store A direct memory

STAX     B              Store A indirect using BC

STAX     D              Store A indirect using DE

STC                     Set carry

XRA      <reg>          Exclusive OR A with register

XRA      M              Exclusive Or A with indirect memory (HL)

XRI      <data>         Exclusive Or A with immediate data

XCHG                    Exchange DE with HL

XTHL                    Exchange HL with top of stack

See  the   manufacturer's   data  sheets   for   more   information.

TASM TABLES  Version 2.8                                     Page 15

Z80 INSTRUCTIONS AND ADDRESSING MODES

The following list shows the acceptable opcode mnemonics  and  their

corresponding   operand  formats for the Z80  version of TASM.   The

following symbols are used in the table:

SYMBOLIC            DESCRIPTION

-----------------------------------------------

<addr>              Absolute address (16 bits)

<bit>               Bit address

<data>              Immediate data (8 bits)

<data16>            Immediate data (16 bits)

<disp>              Relative address

<reg>               Register (A, B, C, D, E, H, or L)

<rp>                Register pair (BC, DE, HL, or SP)

<port>              Port (0 - 255)

<cond>              Condition

NZ - not zero

Z  - zero

NC - not carry

C  - carry

PO - parity odd

PE - parity even

P  - positive

M  - minus

Any valid TASM expression can appear in the  place  of the   <addr>,

<bit>, <data>, <data16>, or <disp> symbolics.

OPCODE  OPERAND         DESCRIPTION

--------------------------------------------------------------------

ADC  A,<data>           Add immediate with carry to accumulator

ADC  A,<reg>            Add register with carry to accumulator

ADC  A,(HL)             Add indirect memory with carry to accumulator

ADC  A,(IX+<disp>)      Add indirect memory with carry to accumulator

ADC  A,(IY+<disp>)      Add indirect memory with carry to accumulator

ADC  HL,<rp>            Add register pair with carry to HL

ADD  A,<data>           Add immediate to accumulator

ADD  A,<reg>            Add register to accumulator

ADD  A,(HL)             Add indirect memory to accumulator

ADD  A,(IX+<disp>)      Add indirect memory to accumulator

ADD  A,(IY+<disp>)      Add indirect memory to accumulator

ADD  HL,<rp>            Add register pair to HL

ADD  IX,<rp>            Add register pair to index register

ADD  IY,<rp>            Add register pair to index register

AND  <data>             And immediate with accumulator

AND  <reg>              And register  with accumulator

AND  (HL)               And memory with accumulator

AND  (IX+<disp>)        And memory with accumulator

AND  (IY+<disp>)        And memory with accumulator

BIT  <bit>,<reg>        Test <bit> in register

BIT  <bit>,(HL)         Test <bit> in indirect memory

BIT  <bit>,(IY+<disp>)  Test <bit> in indirect memory

BIT  <bit>,(IX+<disp>)  Test <bit> in indirect memory

TASM TABLES  Version 2.8                                     Page 16

CALL <addr>             Call the routine at <addr>

CALL <cond>,<addr>      Call the routine if <cond> is satisfied

CCF                     Complement carry flag

CP   <data>             Compare immediate data with accumulator

CP   <reg>              Compare register with accumulator

CP   (HL)               Compare indirect memory with accumulator

CP   (IX+<disp>)        Compare indirect memory with accumulator

CP   (IY+<disp>)        Compare indirect memory with accumulator

CPD                     Compare accumulator with memory and

decrement address and byte counters

CPDR                    Compare accumulator with memory and

decrement address and byte counter,

continue until match is found or

byte counter is zero

CPI                     Compare accumulator with memory and

increment address and byte counters

CPIR                    Compare accumulator with memory and

increment address and byte counter,

continue until match is found or

byte counter is zero

CPL                     Complement the accumulator

DAA                     Decimal adjust accumulator

DEC  <reg>              Decrement register contents

DI                      Disable interrupts

DJNZ <disp>             Decrement reg B and jump relative if zero

EI                      Enable interrupts

EX   AF,AF'             Exchange program status and alt program stat

EX   DE,HL              Exchange DE and HL contents

EX   (SP),HL            Exchange contents of HL and top of stack

EX   (SP),IX            Exchange contents of IX and top of stack

EX   (SP),IY            Exchange contents of IY and top of stack

EXX                     Exchange register pairs and alt reg pairs

HALT                    Program execution stops

IM   0                  Interrupt mode 0

IM   1                  Interrupt mode 1

IM   2                  Interrupt mode 2

IN   A,<port>           Input port to accumulator

INC  <reg>              Increment contents of register

INC  <rp>               Increment contents of register pair

INC  IX                 Increment IX

INC  IY                 Increment IY

INC  (HL)               Increment indirect memory

INC  (IX+<disp>)        Increment indirect memory

INC  (IY+<disp>)        Increment indirect memory

IND                     Input to memory and decrement pointer

INDR                    Input to memory and decrement pointer until

byte counter is zero

INI                     Input to memory and increment pointer

INIR                    Input to memory and increment pointer until

byte counter is zero

IN   <reg>,(C)          Input to register

JP   <addr>             Jump to location

TASM TABLES  Version 2.8                                     Page 17

JP   <cond>,<addr>      Jump to location if condition satisifed

JP   (HL)               Jump to location pointed to by HL

JP   (IX)               Jump to location pointed to by IX

JP   (IY)               Jump to location pointed to by IY

JR   <disp>             Jump relative

JR   C,<disp>           Jump relative if carry is set

JR   NC,<disp>          Jump relative if carry bit is reset

JR   NZ,<disp>          Jump relative if zero flag is reset

JR   Z,<disp>           Jump relative if zero flag is set

LD   A,I                Move interrupt vector contents to accumulator

LD   A,R                Move refresh reg contents to accumulator

LD   A,(<addr>)         Load accumulator indirect from memory

LD   A,(<rp>)           Load accumulator indirect from memory by <rp>

LD   <reg>,<reg>        Load source register to destination register

LD   <rp>,(<addr>)      Load register pair indirect from memory

LD   IX,(<addr>)        Load IX indirect from memory

LD   IY,(<addr>)        Load IY indirect from memory

LD   I,A                Load interrup vector from accumulator

LD   R,A                Load refresh register from accumulator

LD   <reg>,<data>       Load register with immediate data

LD   <rp>,<data16>      Load register pair with immediate data

LD   IX,<data16>        Load  IX  with immediate data

LD   IY,<data16>        Load  IY  with immediate data

LD   <reg>,(HL)         Load register indirect from memory

LD   <reg>,(IX+<disp>)  Load register indirect from memory

LD   <reg>,(IY+<disp>)  Load register indirect from memory

LD   SP,HL              Load contents of HL to stack pointer

LD   SP,IX              Load contents of IX to stack pointer

LD   SP,IY              Load contents of IY to stack pointer

LD   (addr),A           Load contents of A to memory

LD   (<addr>),HL        Load contents of HL to memory

LD   (<addr>),<rp>      Load contents of register pair to memory

LD   (<addr>),IX        Load contents of IX to memory

LD   (<addr>),IY        Load contents of IY to memory

LD   (HL),<data>        Load immediate into indirect memory

LD   (IX+<disp>),<data> Load immediate into indirect memory

LD   (IY+<disp>),<data> Load immediate into indirect memory

LD   (HL),<reg>         Load register  into indirect memory

LD   (IX+<disp>),<reg>  Load register  into indirect memory

LD   (IY+<disp>),<reg>  Load register  into indirect memory

LD   (<rp>),A           Load accumulator into indirect memory

LDD                     Transfer data between memory and decrement

destination and source addresses

LDDR                    Transfer data between memory until byte

counter is zero, decrement destintation

and source addresses

LDI                     Transfer data between memory and increment

destination and source addresses

LDIR                    Transfer data between memory until byte

counter is zero, increment destination

and source addresses

NEG                     Negate contents of accumulator

NOP                     No operation

OR   <data>             Or immediate with accumulator

TASM TABLES  Version 2.8                                     Page 18

OR   <reg>              Or register with accumulator

OR   (HL)               Or indirect memory with accumulator

OR   (IX+<disp>)        Or indirect memory with accumulator

OR   (IY+<disp>)        Or indirect memory with accumulator

OUT  (C),<reg>          Output from registor

OUTD                    Output from memory, decrement address

OTDR                    Output from memory, decrement address

continue until reg B is zero

OUTI                    Output from memory, increment address

OTIR                    Output from memory, increment address

continue until reg B is zero

OUT  <port>,A           Output from accumulator

POP  <rp>               Load register pair from top of stack

POP  IX                 Load IX from top of stack

POP  IY                 Load IY from top of stack

PUSH <rp>               Store resister pair on top of stack

PUSH IX                 Store IX on top of stack

PUSH IY                 Store IY on top of stack

RES  <bit>,<reg>        Reset register bit

RES  <bit>,(HL)         Reset bit at indirect memory location

RES  <bit>,(IX+disp)    Reset bit at indirect memory location

RES  <bit>,(IY+<disp>)  Reset bit at indirect memory location

RET                     Return from subroutine

RET  <cond>             Return from subroutine if condition true

RETI                    Return from interrupt

RETN                    Return from non-maskable interrupt

RL   <reg>              Rotate left through carry register contents

RL   (HL)               Rotate left through carry indirect memory

RL   (IX+<disp>)        Rotate left through carry indirect memory

RL   (IY+<disp>)        Rotate left through carry indirect memory

RLA                     Rotate left through carry accumulator

RLC  <reg>              Rotate left branch  carry register contents

RLC  (HL)               Rotate left branch  carry indirect memory

RLC  (IX+<disp>)        Rotate left branch  carry indirect memory

RLC  (IY+<disp>)        Rotate left branch  carry indirect memory

RLCA                    Rotate left accumulator

RLD                     Rotate one BCD digit left between the

accumulator and memory

RR   <reg>              Rotate right through carry register contents

RR   (HL)               Rotate right through carry indirect memory

RR   (IX+<disp>)        Rotate right through carry indirect memory

RR   (IY+<disp>)        Rotate right through carry indirect memory

RRA                     Rotate right through carry accumulator

RRC  <reg>              Rotate right branch  carry register contents

RRC  (HL)               Rotate right branch  carry indirect memory

RRC  (IX+<disp>)        Rotate right branch  carry indirect memory

RRC  (IY+<disp>)        Rotate right branch  carry indirect memory

RRCA                    Rotate right branch  carry accumulator

RRD                     Rotate one BCD digit right between the

accumulator and memory

RST                     Restart

SBC  A,<data>           Subtract data            from A with borrow

SBC  A,<reg>            Subtract register        from A with borrow

SBC  A,(HL)             Subtract indirect memory from A with borrow

SBC  A,(IX+<disp>)      Subtract indirect memory from A with borrow

SBC  A,(IY+<disp>)      Subtract indirect memory from A with borrow

TASM TABLES  Version 2.8                                     Page 19

SBC  HL,<rp>            Subtract register pair from HL with borrow

SCF                     Set carry flag

SET  <bit>,<reg>        Set register bit

SET  <bit>,(HL)         Set indirect memory bit

SET  <bit>,(IX+<disp>)  Set indirect memory bit

SET  <bit>,(IY+<disp>)  Set indirect memory bit

SLA  <reg>              Shift register left arithmetic

SLA  (HL)               Shift indirect memory left arithmetic

SLA  (IX+<disp>)        Shift indirect memory left arithmetic

SLA  (IY+<disp>)        Shift indirect memory left arithmetic

SRA  <reg>              Shift register right arithmetic

SRA  (HL)               Shift indirect memory right arithmetic

SRA  (IX+<disp>)        Shift indirect memory right arithmetic

SRA  (IY+<disp>)        Shift indirect memory right arithmetic

SRL  <reg>              Shift register right logical

SRL  (HL)               Shift indirect memory right logical

SRL  (IX+<disp>)        Shift indirect memory right logical

SRL  (IY+<disp>)        Shift indirect memory right logical

SUB  <data>             Subtract immediate from accumulator

SUB  <reg>              Subtract register from accumulator

SUB  (HL)               Subtract indirect memory from accumulator

SUB  (IX+<disp>)        Subtract indirect memory from accumulator

SUB  (IY+<disp>)        Subtract indirect memory from accumulator

XOR  <data>             Exclusive or immediate with accumulator

XOR  <reg>              Exclusive or register with accumulator

XOR  (HL)               Exclusive or indirect memory with accumulator

XOR  (IX+<disp>)        Exclusive or indirect memory with accumulator

XOR  (IY+<disp>)        Exclusive or indirect memory with accumulator

See  the   manufacturer's   data  sheets   for   more   information.

TASM TABLES  Version 2.8                                     Page 20

6805 INSTRUCTIONS AND ADDRESSING MODES

The following list shows the acceptable opcode mnemonics  and  their

corresponding  operand  formats for the 6805  version of TASM.   The

following symbols are used in the table:

SYMBOLIC            DESCRIPTION

-----------------------------------------------

<addr>              Absolute address (16 bits)

<addr8>             Absolute address (8 bits)

<bit>               Bit address

<data>              Immediate data (8 bits)

<rel>               Relative address

Any valid TASM expression can appear in the  place  of the   <addr>,

<addr8>, <bit>, <data>, or <rel> symbolics.

OPCODE  OPERAND         DESCRIPTION

--------------------------------------------------------------

ADC     #<data>         Add with carry, immediate

ADC     ,X              Add with carry, indexed, no offset

ADC     <addr8>,X       Add with carry, indexed, 1 byte offset

ADC     <addr>,X        Add with carry, indexed, 2 byte offset

ADC     <addr8>         Add with carry, direct

ADC     <addr>          Add with carry, extended

ADD     #<data>         Add, immediate

ADD     ,X              Add, indexed, no offset

ADD     <addr8>,X       Add, indexed, 1 byte offset

ADD     <addr>,X        Add, indexed, 2 byte offset

ADD     <addr8>         Add, direct

ADD     <addr>          Add, extended

AND     #<data>         And, immediate

AND     ,X              And, indexed, no offset

AND     <addr8>,X       And, indexed, 1 byte offset

AND     <addr>,X        And, indexed, 2 byte offset

AND     <addr8>         And, direct

AND     <addr>          And, extended

ASLA                    Arithmetic Shift Left, accumulator

ASLX                    Arithmetic Shift Left, index register

ASL     <addr8>         Arithmetic Shift Left, direct

ASL     ,X              Arithmetic Shift Left, indexed, no offset

ASL     <addr8>,X       Arithmetic Shift Left, indexed, 1 byte offset

ASRA                    Arithmetic Shift Right, accumulator

ASRX                    Arithmetic Shift Right, index register

ASR     <addr8>         Arithmetic Shift Right, direct

ASR     ,X              Arithmetic Shift Right, indexed, no offset

ASR     <addr8>,X       Arithmetic Shift Right, indexed, 1 byte offset

BCC     <rel>           Branch if carry clear

BCLR    <bit>,<addr8>   Bit Clear in memory

BCS     <rel>           Branch if carry set

BEQ     <rel>           Branch if equal

TASM TABLES  Version 2.8                                     Page 21

BHCC    <rel>           Branch if half carry clear

BHCS    <rel>           Branch if half carry set

BHI     <rel.           Branch if higher

BHS     <rel>           Branch if higher or same

BIH     <rel>           Branch if interrupt line is high

BIL     <rel>           Branch if interrupt is low

BIT     #<data>         Bit test, immediate

BIT     ,X              Bit test, indexed, no offset

BIT     <addr8>,X       Bit test, indexed, 1 byte offset

BIT     <addr>,X        Bit test, indexed, 2 byte offset

BIT     <addr8>         Bit test, direct

BIT     <addr>          Bit test, extended

BLO     <rel>           Branch if lower

BLS     <rel>           Branch if lower or same

BMC     <rel>           Branch if interrupt mask is clear

BMI     <rel>           Branch if minus

BMS     <rel>           Branch if interuupt mask bit is set

BNE     <rel>           Branch if not equal

BPL     <rel>           Branch if plus

BRA     <rel>           Branch always

BRCLR   <bit>,<addr8>,<rel>     Branch if bit is clear

BRN     <rel>           Branch never

BRSET   <bit>,<addr8>,<rel>     Branch if bit is set

BSET    <bit>,<addr8>   Bit set in memory

BSR     <rel>           Branch to subroutine

CLC                     Clear carry bit

CLI                     Clear interuupt mask bit

CLRA                    Clear, accumulator

CLRX                    Clear, index register

CLR     <addr8>         Clear, direct

CLR     ,X              Clear, indexed, no offset

CLR     <addr8>,X       Clear, indexed, 1 byte offset

CMP     #<data>         Compare Acc, immediate

CMP     ,X              Compare Acc, indexed, no offset

CMP     <addr8>,X       Compare Acc, indexed, 1 byte offset

CMP     <addr>,X        Compare Acc, indexed, 2 byte offset

CMP     <addr8>         Compare Acc, direct

CMP     <addr>          Compare Acc, extended

COMA                    Complement, accumulator

COMX                    Complement, index register

COM     <addr8>         Complement, direct

COM     ,X              Complement, indexed, no offset

COM     <addr8>,X       Complement, indexed, 1 byte offset

CPX     #<data>         Compare Index, immediate

CPX     ,X              Compare Index, indexed, no offset

CPX     <addr8>,X       Compare Index, indexed, 1 byte offset

CPX     <addr>,X        Compare Index, indexed, 2 byte offset

CPX     <addr8>         Compare Index, direct

CPX     <addr>          Compare Index, extended

TASM TABLES  Version 2.8                                     Page 22

DECA                    Decrement, accumulator

DECX                    Decrement, index register

DEX                     Decrement, index register (alternate of DECX)

DEC     <addr8>         Decrement, direct

DEC     ,X              Decrement, indexed, no offset

DEC     <addr8>,X       Decrement, indexed, 1 byte offset

EOR     #<data>         Exclusive OR, immediate

EOR     ,X              Exclusive OR, indexed, no offset

EOR     <addr8>,X       Exclusive OR, indexed, 1 byte offset

EOR     <addr>,X        Exclusive OR, indexed, 2 byte offset

EOR     <addr8>         Exclusive OR, direct

EOR     <addr>          Exclusive OR, extended

INCA                    Increment, accumulator

INCX                    Increment, index register

INX                     Increment, index register (alternate of INCX)

INC     <addr8>         Increment, direct

INC     ,X              Increment, indexed, no offset

INC     <addr8>,X       Increment, indexed, 1 byte offset

JMP     ,X              Jump, indexed, no offset

JMP     <addr8>,X       Jump, indexed, 1 byte offset

JMP     <addr>,X        Jump, indexed, 2 byte offset

JMP     <addr8>         Jump, direct

JMP     <addr>          Jump, extended

JSR     ,X              Jump Subroutine, indexed, no offset

JSR     <addr8>,X       Jump Subroutine, indexed, 1 byte offset

JSR     <addr>,X        Jump Subroutine, indexed, 2 byte offset

JSR     <addr8>         Jump Subroutine, direct

JSR     <addr>          Jump Subroutine, extended

LDA     #<data>         Load Acc, immediate

LDA     ,X              Load Acc, indexed, no offset

LDA     <addr8>,X       Load Acc, indexed, 1 byte offset

LDA     <addr>,X        Load Acc, indexed, 2 byte offset

LDA     <addr8>         Load Acc, direct

LDA     <addr>          Load Acc, extended

LDX     #<data>         Load Index, immediate

LDX     ,X              Load Index, indexed, no offset

LDX     <addr8>,X       Load Index, indexed, 1 byte offset

LDX     <addr>,X        Load Index, indexed, 2 byte offset

LDX     <addr8>         Load Index, direct

LDX     <addr>          Load Index, extended

LSLA                    Logical Shift Left, accumulator

LSLX                    Logical Shift Left, index register

LSL     <addr8>         Logical Shift Left, direct

LSL     ,X              Logical Shift Left, indexed, no offset

LSL     <addr8>,X       Logical Shift Left, indexed, 1 byte offset

LSRA                    Logical Shift Right, accumulator

LSRX                    Logical Shift Right, index register

LSR     <addr8>         Logical Shift Right, direct

TASM TABLES  Version 2.8                                     Page 23

LSR     ,X              Logical Shift Right, indexed, no offset

LSR     <addr8>,X       Logical Shift Right, indexed, 1 byte offset

NEGA                    Negate, accumulator

NEGX                    Negate, index register

NEG     <addr8>         Negate, direct

NEG     ,X              Negate, indexed, no offset

NEG     <addr8>,X       Negate, indexed, 1 byte offset

NOP                     No Operation

ORA     #<data>         Inclusive OR Acc, immediate

ORA     ,X              Inclusive OR Acc, indexed, no offset

ORA     <addr8>,X       Inclusive OR Acc, indexed, 1 byte offset

ORA     <addr>,X        Inclusive OR Acc, indexed, 2 byte offset

ORA     <addr8>         Inclusive OR Acc, direct

ORA     <addr>          Inclusive OR Acc, extended

ROLA                    Rotate Left thru Carry, accumulator

ROLX                    Rotate Left thru Carry, index register

ROL     <addr8>         Rotate Left thru Carry, direct

ROL     ,X              Rotate Left thru Carry, indexed, no offset

ROL     <addr8>,X       Rotate Left thru Carry, indexed, 1 byte offset

RORA                    Rotate Right thru Carry, accumulator

RORX                    Rotate Right thru Carry, index register

ROR     <addr8>         Rotate Right thru Carry, direct

ROR     ,X              Rotate Right thru Carry, indexed, no offset

ROR     <addr8>,X       Rotate Right thru Carry, indexed, 1 byte offset

RSP                     Reset Stack Pointer

RTI                     Return from Interrupt

RTS                     Return from Subroutine

SBC     #<data>         Subtract with Carry, immediate

SBC     ,X              Subtract with Carry, indexed, no offset

SBC     <addr8>,X       Subtract with Carry, indexed, 1 byte offset

SBC     <addr>,X        Subtract with Carry, indexed, 2 byte offset

SBC     <addr8>         Subtract with Carry, direct

SBC     <addr>          Subtract with Carry, extended

SEC                     Set carry bit

SEI                     Set interrupt Mask bit

STA     #<data>         Store Acc, immediate

STA     ,X              Store Acc, indexed, no offset

STA     <addr8>,X       Store Acc, indexed, 1 byte offset

STA     <addr>,X        Store Acc, indexed, 2 byte offset

STA     <addr8>         Store Acc, direct

STA     <addr>          Store Acc, extended

STOP                    Enable IRQ, Stop Oscillator

STX     #<data>         Store Index, immediate

STX     ,X              Store Index, indexed, no offset

STX     <addr8>,X       Store Index, indexed, 1 byte offset

TASM TABLES  Version 2.8                                     Page 24

STX     <addr>,X        Store Index, indexed, 2 byte offset

STX     <addr8>         Store Index, direct

STX     <addr>          Store Index, extended

SUB     #<data>         Subtract, immediate

SUB     ,X              Subtract, indexed, no offset

SUB     <addr8>,X       Subtract, indexed, 1 byte offset

SUB     <addr>,X        Subtract, indexed, 2 byte offset

SUB     <addr8>         Subtract, direct

SUB     <addr>          Subtract, extended

SWI                     Software Interrupt

TAX                     Transfer Acc to Index

TSTA                    Test for neg or zero, accumulator

TSTX                    Test for neg or zero, index register

TST     <addr8>         Test for neg or zero, direct

TST     ,X              Test for neg or zero, indexed, no offset

TST     <addr8>,X       Test for neg or zero, indexed, 1 byte offset

TXA                     Transfer Index to Acc

WAIT                    Enable Interrupt, Stop Processor

See  the   manufacturer's   data  sheets   for   more   information.

TASM TABLES  Version 2.8                                     Page 25

TMS32010 INSTRUCTIONS AND ADDRESSING MODES

The following list shows the acceptable opcode mnemonics  and  their

corresponding   operand  formats for the TMS32010   version of TASM.

The following symbols are used in the table:

SYMBOLIC            DESCRIPTION

-----------------------------------------------

<ar>                Auxiliary register (AR0, AR1)

<arp>               Auxiliary register pointer

<dma>               Direct memory address

<pma>               Program memory address

<port>              Port address (0 - 7)

<shift>             Shift count  (0 - 15)

<const1>            Constant (1 bit)

<const8>            Constant (8 bit)

<const13>           Constant (13 bit)

Any valid TASM expression can appear in the  place  of  any  of  the

above symbolics.

OPCODE   OPERAND            DESCRIPTION

--------------------------------------------------------------------

ABS                         Absolute value of ACC

ADD      *+,<shift>,<arp>   Add to ACC with shift

ADD      *-,<shift>,<arp>

ADD      *, <shift>,<arp>

ADD      *+,<shift>

ADD      *-,<shift>

ADD      *, <shift>

ADD      *+

ADD      *-

ADD      *

ADD      <dma>,<shift>

ADD      <dma>

ADDH     *+,<arp>           Add to high-order ACC bits

ADDH     *-,<arp>

ADDH     *, <arp>

ADDH     *+

ADDH     *-

ADDH     *

ADDH     <dma>

ADDS     *+,<arp>           Add to ACC with no sign extension

ADDS     *-,<arp>

ADDS     *, <arp>

ADDS     *+

ADDS     *-

ADDS     *

ADDS     <dma>

AND      *+,<arp>           AND with ACC

AND      *-,<arp>

AND      *, <arp>

TASM TABLES  Version 2.8                                     Page 26

AND      *+

AND      *-

AND      *

AND      <dma>

APAC                        Add P register to ACC

B        <pma>              Branch unconditionally

BANZ     <pma>              Branch on auxiliary register not zero

BGEZ     <pma>              Branch if ACC >= 0

BGZ      <pma>              Branch if ACC >  0

BIOZ     <pma>              Branch on BIO- = 0

BLEZ     <pma>              Branch if ACC <= 0

BLZ      <pma>              Branch if ACC <  0

BNZ      <pma>              Branch if ACC <> 0

BV       <pma>              Branch on overflow

BZ       <pma>              Branch if ACC =  0

CALA                        Call subroutine from ACC

CALL     <pma>              Call subroutine at <pma>

DINT                        Disable interrupt

DMOV     *+,<arp>           Data move in memory

DMOV     *-,<arp>

DMOV     *, <arp>

DMOV     *+

DMOV     *-

DMOV     *

DMOV     <dma>

EINT                        Enable Interrupt

IN       *+,<port> ,<arp>   Input data from port

IN       *-,<port> ,<arp>

IN       * ,<port> ,<arp>

IN       *+,<port>

IN       *-,<port>

IN       * ,<port>

IN       <dma>,<port>

LAC      *+,<shift>,<arp>   Load ACC with shift

LAC      *-,<shift>,<arp>

LAC      *, <shift>,<arp>

LAC      *+,<shift>

LAC      *-,<shift>

LAC      *, <shift>

LAC      *+

LAC      *-

LAC      *

LAC      <dma>,<shift>

LAC      <dma>

LACK     <const8>                   Load ACC with 8 bit constant

LAR      <ar>,*+,<arp>              Load auxiliary Register

TASM TABLES  Version 2.8                                     Page 27

LAR      <ar>,*-,<arp>

LAR      <ar>,*, <arp>

LAR      <ar>,*+

LAR      <ar>,*-

LAR      <ar>,*

LAR      <ar>,<dma>

LARK     <ar>,<const8>              Load aux register with constant

LARP     <const1>                   Load aux register pointer immed

LDP      *+,<arp>                   Load data memory page pointer

LDP      *-,<arp>

LDP      *, <arp>

LDP      *+

LDP      *-

LDP      *

LDP      <dma>

LDPK     <const1>                   Load data page pointer immediate

LST      *+,<arp>                   Load status from data memory

LST      *-,<arp>

LST      *, <arp>

LST      *+

LST      *-

LST      *

LST      <dma>

LT       *+,<arp>                   Load T register

LT       *-,<arp>

LT       *, <arp>

LT       *+

LT       *-

LT       *

LT       <dma>

LTA      *+,<arp>                   Load T register and accumulate

LTA      *-,<arp>                     product

LTA      *, <arp>

LTA      *+

LTA      *-

LTA      *

LTA      <dma>

LTD      *+,<arp>                   Load T reg, accumulate product,

LTD      *-,<arp>                     and move

LTD      *, <arp>

LTD      *+

LTD      *-

LTD      *

LTD      <dma>

MAR      *+,<arp>                   Modify auxiliary register

MAR      *-,<arp>

MAR      *, <arp>

MAR      *+

TASM TABLES  Version 2.8                                     Page 28

MAR      *-

MAR      *

MAR      <dma>

MPY      *+,<arp>                   Multiply

MPY      *-,<arp>

MPY      *, <arp>

MPY      *+

MPY      *-

MPY      *

MPY      <dma>

MPYK     <const13>                  Multiply immediate

NOP                                 No Operation

OR       *+,<arp>                   OR  with low order bits of ACC

OR       *-,<arp>

OR       *, <arp>

OR       *+

OR       *-

OR       *

OR       <dma>

OUT      *+,<port>,<arp>            Output data to port

OUT      *-,<port>,<arp>

OUT      *, <port>,<arp>

OUT      *+,<port>

OUT      *-,<port>

OUT      *, <port>

OUT      <dma>,<port>

PAC                                 Load ACC with P register

POP                                 Pop top of stack to ACC

PUSH                                Push ACC onto stack

RET                                 Return from subroutine

ROVM                                Reset overflow mode register

SACH     *+,<shift>,<arp>           Store ACC high with shift

SACH     *-,<shift>,<arp>             Note: shift can only be 0, 1,

SACH     *, <shift>,<arp>                   or 4

SACH     *+,<shift>

SACH     *-,<shift>

SACH     *, <shift>

SACH     *+

SACH     *-

SACH     *

SACH     <dma>,<shift>

SACH     <dma>

SACL     *+,<arp>                   Store ACC low

SACL     *-,<arp>

SACL     *, <arp>

SACL     *+

SACL     *-

SACL     *

TASM TABLES  Version 2.8                                     Page 29

SACL     <dma>

SAR      <ar>,*+,<arp>              Store auxiliary Register

SAR      <ar>,*-,<arp>

SAR      <ar>,*, <arp>

SAR      <ar>,*+

SAR      <ar>,*-

SAR      <ar>,*

SAR      <ar>,<dma>

SOVM                                Set overflow mode register

SPAC                                Subtract P register from ACC

SST      *+,<arp>                   Store status

SST      *-,<arp>

SST      *, <arp>

SST      *+

SST      *-

SST      *

SST      <dma>

SUB      *+,<shift>,<arp>           Subtract from ACC with shift

SUB      *-,<shift>,<arp>

SUB      *, <shift>,<arp>

SUB      *+,<shift>

SUB      *-,<shift>

SUB      *, <shift>

SUB      *+

SUB      *-

SUB      *

SUB      <dma>,<shift>

SUB      <dma>

SUBC     *+,<arp>                   Conditional subtract

SUBC     *-,<arp>

SUBC     *, <arp>

SUBC     *+

SUBC     *-

SUBC     *

SUBC     <dma>

SUBH     *+,<arp>                   Subtract from high-order ACC

SUBH     *-,<arp>

SUBH     *, <arp>

SUBH     *+

SUBH     *-

SUBH     *

SUBH     <dma>

SUBS     *+,<arp>                   Subtract from low ACC with

SUBS     *-,<arp>                     sign-extension suppressed

SUBS     *, <arp>

SUBS     *+

SUBS     *-

SUBS     *

SUBS     <dma>

TASM TABLES  Version 2.8                                     Page 30

TBLR     *+,<arp>                   Table Read

TBLR     *-,<arp>

TBLR     *, <arp>

TBLR     *+

TBLR     *-

TBLR     *

TBLR     <dma>

TBLW     *+,<arp>                   Table Write

TBLW     *-,<arp>

TBLW     *, <arp>

TBLW     *+

TBLW     *-

TBLW     *

TBLW     <dma>

XOR      *+,<arp>                   Exclusive OR with low bits of ACC

XOR      *-,<arp>

XOR      *, <arp>

XOR      *+

XOR      *-

XOR      *

XOR      <dma>

ZAC                                 Zero the ACC

ZALH     *+,<arp>                   Zero ACC and load high

ZALH     *-,<arp>

ZALH     *, <arp>

ZALH     *+

ZALH     *-

ZALH     *

ZALH     <dma>

ZALS     *+,<arp>                   Zero ACC and load low with

ZALS     *-,<arp>                     sign extension suppressed

ZALS     *, <arp>

ZALS     *+

ZALS     *-

ZALS     *

ZALS     <dma>

See manufacturer's data for more information.

TASM TABLES  Version 2.8                                     Page 31

TMS7000 INSTRUCTIONS AND ADDRESSING MODES

The following list shows the acceptable opcode mnemonics  and  their

corresponding  operand   formats  for the TMS7000  version of  TASM.

The following symbolic fields used in the table:

SYMBOLIC        DESCRIPTION

-------------------------------------------

<iop>           Immediate data (8 bits)

<Rn>            Register file (memory locations 0 to 127 or

0 to 255 depending on on-chip RAM)

<Pn>            Peripheral file (0-255)

<rel>           Program address (relative)

<addr>          Program address (16 bit)

<trap>          Trap number (0-23)

Any valid TASM expression can appear in the  place  of  any  of  the

above symbolics.

Note  that   TASM  allows   an  alternate   syntax  for   expressing

indirection.  Parenthesis can be  replaced with brackets (which  are

less ambiguous because they do not occur in expressions).  Thus, the

following are equivalent:

BR      @addr1(B)

BR      @addr1[B]

OPCODE  OPERANDS

---------------------------------------

ADC    B,A

ADC    %<iop>,A

ADC    %<iop>,B

ADC    %<iop>,<Rn>

ADC    <Rn>,A

ADC    <Rn>,B

ADC    <Rn>,<Rn>

ADD    B,A

ADD    %<iop>,A

ADD    %<iop>,B

ADD    %<iop>,<Rn>

ADD    <Rn>,A

ADD    <Rn>,B

ADD    <Rn>,<Rn>

AND    B,A

AND    %<iop>,A

AND    %<iop>,B

AND    %<iop>,<Rn>

AND    <Rn>,A

AND    <Rn>,B

AND    <Rn>,<Rn>

ANDP   A,<Pn>

ANDP   B,<Pn>

TASM TABLES  Version 2.8                                     Page 32

ANDP   %<iop>,<Pn>

BTJO   B,A,<rel>

BTJO   %<iop>,A,<rel>

BTJO   %<iop>,B,<rel>

BTJO   %<iop>,<Rn>,<rel>

BTJO   <Rn>,A,<rel>

BTJO   <Rn>,B,<rel>

BTJO   <Rn>,<Rn>,<rel>

BTJOP  A,<Pn>,<rel>

BTJOP  B,<Pn>,<rel>

BTJOP  %<iop>,<Pn>,<rel>

BTJZ   B,A,<rel>

BTJZ   %<iop>,A,<rel>

BTJZ   %<iop>,B,<rel>

BTJZ   %<iop>,<Rn>,<rel>

BTJZ   <Rn>,A,<rel>

BTJZ   <Rn>,B,<rel>

BTJZ   <Rn>,<Rn>,<rel>

BTJZP  A,<Pn>,<rel>

BTJZP  B,<Pn>,<rel>

BTJZP  %<iop>,<Pn>,<rel>

BR      @<addr>(B)

BR      @<addr>[B]

BR      @<addr>

BR      *<Rn>

CALL    @<addr>(B)

CALL    @<addr>[B]

CALL    @<addr>

CALL    *<Rn>

CLR     A

CLR     B

CLR     <Rn>

CLRC

CMP     B,A

CMP     %<iop>,A

CMP     %<iop>,B

CMP     %<iop>,<Rn>

CMP     <Rn>,A

CMP     <Rn>,B

CMP     <Rn>,<Rn>

CMPA    @<addr>(B)

CMPA    @<addr>[B]

CMPA    @<addr>

CMPA    *<Rn>

DAC     B,A

DAC     %<iop>,A

TASM TABLES  Version 2.8                                     Page 33

DAC     %<iop>,B

DAC     %<iop>,<Rn>

DAC     <Rn>,A

DAC     <Rn>,B

DAC     <Rn>,<Rn>

DEC     A

DEC     B

DEC     <Rn>

DECD    A

DECD    B

DECD    <Rn>

DINT

DJNZ    A,<rel>

DJNZ    B,<rel>

DJNZ    <Rn>,<rel>

DSB     B,A

DSB     %<iop>,A

DSB     %<iop>,B

DSB     %<iop>,<Rn>

DSB     <Rn>,A

DSB     <Rn>,B

DSB     <Rn>,<Rn>

EINT

IDLE

INC     A

INC     B

INC     <Rn>

INV     A

INV     B

INV     <Rn>

JMP     <rel>

JC      <rel>

JEQ     <rel>

JGE     <rel>

JGT     <rel>

JHS     <rel>

JL      <rel>

JN      <rel>

JNC     <rel>

JNE     <rel>

JNZ     <rel>

JP      <rel>

JPZ     <rel>

JZ      <rel>

LDA     @<addr>(B)

TASM TABLES  Version 2.8                                     Page 34

LDA     @<addr>[B]

LDA     @<addr>

LDA     *<Rn>

LDSP

MOV     A,B

MOV     B,A

MOV     A,<Rn>

MOV     B,<Rn>

MOV     %<iop>,A

MOV     %<iop>,B

MOV     %<iop>,<Rn>

MOV     <Rn>,A

MOV     <Rn>,B

MOV     <Rn>,<Rn>

MOVD    %<iop>[B],<Rn>

MOVD    %<iop>,<Rn>

MOVD    <Rn>,<Rn>

MOVP    A,<Pn>

MOVP    B,<Pn>

MOVP    %<iop>,<Pn>

MOVP    <Pn>,A

MOVP    <Pn>,B

MPY     B,A

MPY     %<iop>,A

MPY     %<iop>,B

MPY     %<iop>,<Rn>

MPY     <Rn>,A

MPY     <Rn>,B

MPY     <Rn>,<Rn>

NOP

OR      B,A

OR      %<iop>,A

OR      %<iop>,B

OR      %<iop>,<Rn>

OR      <Rn>,A

OR      <Rn>,B

OR      <Rn>,<Rn>

ORP     A,<Pn>

ORP     B,<Pn>

ORP     %<iop>,<Pn>

POP     A

POP     B

POP     ST

POP     <Rn>

POPST

PUSH    A

TASM TABLES  Version 2.8                                     Page 35

PUSH    B

PUSH    ST

PUSH    <Rn>

PUSHST

RETI

RETS

RL      A

RL      B

RL      <Rn>

RLC     A

RLC     B

RLC     <Rn>

RR      A

RR      B

RR      <Rn>

RRC     A

RRC     B

RRC     <Rn>

SBB     B,A

SBB     %<iop>,A

SBB     %<iop>,B

SBB     %<iop>,<Rn>

SBB     <Rn>,A

SBB     <Rn>,B

SBB     <Rn>,<Rn>

SETC

STA     @<addr>(B)

STA     @<addr>[B]

STA     @<addr>

STA     *<Rn>

STSP

SUB     B,A

SUB     %<iop>,A

SUB     %<iop>,B

SUB     %<iop>,<Rn>

SUB     <Rn>,A

SUB     <Rn>,B

SUB     <Rn>,<Rn>

SWAP    A

SWAP    B

SWAP    <Rn>

TRAP    <trap>

TASM TABLES  Version 2.8                                     Page 36

TST     A

TSTA

TST     B

TSTB

XCHB    A

XCHB    <Rn>

XOR     B,A

XOR     %<iop>,A

XOR     %<iop>,B

XOR     %<iop>,<Rn>

XOR     <Rn>,A

XOR     <Rn>,B

XOR     <Rn>,<Rn>

XORP    A,<Pn>

XORP    B,<Pn>

XORP    %<iop>,<Pn>

TASM TABLES  Version 2.8                                     Page 37

BUILDING TASM FROM THE SOURCE CODE

TASM  can  be built using the provided 'make'  file,  assuming   the

Microsoft C compiler (version 5.0) is available.  The can be done as

follows:

make tasm.mak

For UNIX, try this:

%cc -DUNIX tasm.c macro.c parse.c str.c -o tasm

The header file tasm.h should also be available.

Note  that the UNIX flag is being defined on the command line.  This

causes the following things to happen:

1.  Tasm.h includes somewhat different system include files

appropriate for the UNIX environment.

2.  TASM declares a 64 Kbyte array  in  which  to  hold  the

assembled    opcodes    and    data  in  a  slightly

different way.   MS C must use the 'far' keyword for

such  an  array  to give it a segment  all  its  own

(assuming use of the small memory model).  Most UNIX

environments do not have need for such syntax.

TASM TABLES  Version 2.8                                     Page 38

TASM INSTRUCTION SET TABLE DEFINITION

The tables that control TASM's interpretation of the source file are

read from a file at run time.  The table file name is determined  by

taking  the  numeric option field specified on the TASM command line

and appending it to the string "TASM", then a  ".TAB"  extension  is

added.  Thus, if the following command line is entered:

tasm -51  test.asm

then TASM would read the table file named "TASM51.TAB".

The following rules apply to the structure of the table file:

1.  The  first  line  of  the  file  should contain a string

surrounded by double quotes that should identify the

version of the assembler  table.  This  string  will

appear at the top of each page in the list file.  It

should be limited to 24 characters.

2.  Any line that starts with a '.' is considered a directive.

The following directives are available:

DIRECTIVE       MEANING

----------------------------------------------------

MSFIRST         Generate opcodes MS byte first.

ALTWILD         Use '@' instead of '*' as the

wild card in the table.

3.  Any   line  whose  first  character  is not a '.' and is

not  alphabetic is considered to be a comment and is

discarded.

4.  Any  line  that has an alphabetic character as the first

character is assumed to be an instruction definition

record  and  is  parsed  to   build   the   internal

representation   of   the  instruction  set  tables.

Eight  fields   (separated  by  white  space)    are

expected,  as follows:

Field Name      Description

--------------------------------------------

INSTRUCTION     Instruction Mnemonic

ARGS            Argument definition

OPCODE          Opcode value

NBYTES          Number of bytes

MODOP           Modifier operation

CLASS           Instruction class

SHIFT           Argument left shift count

OR              Argument bitwise OR mask

INSTRUCTION.  The  INSTRUCTION  field should contain

the string to be  used  as  the  mnemonic  for  this

instruction.  Upper case letters should be used (the

source statements are converted to upper case before

comparison).

TASM TABLES  Version 2.8                                     Page 39

ARGS.   The  ARGS  field  should  contain  a  string

describing  the  format  of  the operand field.  All

characters are taken literally except the '*'  which

denotes  the  presence  of  a valid TASM expression.

Multiple '*'s can be used,  but all but the last one

must  be  followed  by  a  comma.  If  a  single '*'

appears in the ARGS field,  then the default  action

of  TASM  will  be  to  determine  the  value of the

expression that matches the field and insert one  or

two  bytes  of  it into the object file depending on

the NBYTES field.  If multiple '*'s are  used,  then

special  operators  (MODOP)  must  be  used  to take

advantage of them (see the examples below).  An ARGS

field of a pair  of  double  quotes  means  that  no

arguments are expected.

OPCODE.  The OPCODE field should contain the  opcode

value    (two  to  six  hex   digits)   for   this

instruction  and  address mode.   Each pair  of  hex

digits  represent  a  single  byte  of  the  opcode,

ordered with the right most pair being placed in the

lowest memory location.

NBYTES.  The NBYTES field should specify the  number

of  bytes  this  instruction  is to occupy (a single

decimal  digit).   This number includes both  opcode

bytes and argument bytes, thus,  the number of bytes

of argument is computed by subtracting the number of

bytes  of  opcode  (dictated by the  length  of  the

OPCODE field) from NBYTES.

MODOP.  The MODOP field determines  if  any  special

operations   need   to  be  performed  on  the  code

generated for this  instruction.  For  example,  the

zero-page  addressing  mode of the 6502 is a special

case of the absolute addressing mode, and is handled

by a special MODOP code (see appendix B).  The  list

of operators is as follows:

TASM TABLES  Version 2.8                                     Page 40

MODOP       DESCRIPTION

---------------------------------------------------

NOTOUCH     Do nothing to instruction or args

JMPPAGE     Put bits 8-10 of first arg into

bits 5-7 of opcode (8048 JMP)

ZPAGE       If arg < 256 then use zero-page (6502)

R1          Make arg relative to PC (single byte)

R2          Make arg relative to PC (two byte)

CREL        Combine LS bytes of first two args

making the second one relative to PC

SWAP        Swap bytes of first arg

COMBINE     Combine LS bytes of first two args into

first arg (arg1 -> LSB, arg2 ->MSB).

CSWAP       Combine LS bytes of first two args into

first arg and swap.

ZBIT        Z80 bit instructions.

MBIT        Motorola (6805) bit instructions

MZERO       Motorola (6805) zero page (direct)

3ARG        Three args, one byte each.

3REL        Three args, one byte each, last one

relative

T1          TMS320 instruction with one arg.  Shift

according to SHIFT and mask with OR and

OR into opcode.  If a second arg exists

assume it is an <arp> and OR into   LSB

of opcode.

TDMA        TMS320 instruction with first arg <dma>.

Second arg gets shift/and/or  treatment

as with T1.

TAR         TMS320 instruction with first arg <ar>.

Second arg gets shift/and/or  treatment

as with T1.

Note  that the reason for the combining of arguments

(COMBINE and CSWAP) is that TASM  assumes  that  all

object  bytes  to be inserted in the object file are

derived from a variable representing  the  value  of

the  first  argument (argval).  If two arguments are

in the  ARGS  field,  then  one  of  the  previously

mentioned  MODOP`s  must  be  used.  They  have  the

effect of combining the low bytes of the  first  two

arguments  into the variable (argval) from which the

object  code  will  be  generated.  TASM`s  argument

parsing   routine  can  handle  a  large  number  of

arguments,  but the code that generates  the  object

code is less capable.

CLASS.  The  CLASS  field is used to specify whether

this instruction is part of the standard instruction

set or a member of a set of  extended  instructions.

Bit 0 of this field should be set to denote a member

of  the standard instruction set.  Other bits can be

used as needed  to  establish  several  classes  (or

sets)   of  instructions  that  can  be  enabled  or

disabled via  the  '-x'  command  line  option  (see

section on 6502 INSTRUCTIONS AND ADDRESSING MODES).

TASM TABLES  Version 2.8                                     Page 41

SHIFT (optional).   The SHIFT field is used to cause

the  first  argument of the given instruction to  be

shifted left the specified number of bits.   (Except

T1, TDMA, TAR MODOPS as noted below).

OR  (optional).   The OR field is used to perform  a

bitwise  OR  with  the first argument of  the  given

instruction.  Specified as hex digits.  (Except  T1,

TDMA, TAR MODOPS as noted below).

Note that the SHIFT/OR fields are used somewhat differently for  T1,

TDMA,  and TAR MODOPS.  In those cases,  the SHIFT and OR fields are

used  but the OR field is really an AND mask and the result is  OR'd

with the opcode.

The   following  table   shows   possible    instruction  definition

records,  followed  by  possible  source statements that would match

it,    followed by the resulting object code that would be generated

(in hex):

EXAMPLE         EXAMPLE

INSTRUCTION DEFINITION                    SOURCE          OBJECT

-------------------------------------------------------------------

XYZ  *      FF   3  NOTOUCH 1             xyz 1234h       FF 34 12

XYZ  *      FF   2  NOTOUCH 1             xyz 1234h       FF 34

ZYX  *      FE   3  SWAP    1             zyx 1234h       FE 12 34

ZYX  *      FE   3  R2      1             zyx $+4         FE 01 00

ABC  *,*    FD   3  COMBINE 1             abc 45h,67h     FD 45 67

ABC  *,*    FD   3  CSWAP   1             abc 45h,67h     FD 67 45

ADD  A,#*   FC   2  NOTOUCH 1             add A,#'B'      FC 42

RET  ""     FB   1  NOTOUCH 1             ret             FB

LD   IX,*   21DD 4  NOTOUCH 1             ld  IX,1234h    DD 21 34 12

LD   IX,*   21DD 4  NOTOUCH 1 1 0         ld  IX,1234h    DD 21 68 24

LD   IX,*   21DD 4  NOTOUCH 1 0 1         ld  IX,1234h    DD 21 35 12

LD   IX,*   21DD 4  NOTOUCH 1 1 1         ld  IX,1234h    DD 21 69 24

LD   IX,*   21DD 4  NOTOUCH 1 8 12        ld  IX,34h      DD 21 12 34

The  order  of  the  entries for various addressing modes of a given

instruction is important.  Since the wild card matches anything,  it

is  important to specify the ARGS for the addressing modes that have

the  most  qualifying  characters  first.   For   example,   if   an

instruction  had  two  addressing  modes,   one  that  accepted  any

expression,  and another that required a pound sign in front  of  an

expression,  the  pound  sign  entry  should  go first otherwise all

occurrences of the instruction would match  the  more  general  ARGS

expression   that  it  encountered  first.   The  following  entries

illustrate the proper sequencing:

ADD #*  12 3 NOTOUCH 1

ADD *   13 3 NOTOUCH 1